# General pump controller operation

Digital Controller  Your digital pump controller has been designed and manufactured to exacting standards and tested fully before leaving the factory. The unit has a warranty for 12 months provided it is fitted and used according to the instructions. We recommend that in all cases the fuse and the pump pressure switch are fitted, as this gives the best possible protection to the pump controller and battery, and also allows the most productive and efficient use of your WFP system. The controller is fitted with the following features as standard:

- Auto calibration

- Dead end (DE) detection (water flow off)

- Pump pressure switch detection

- Voltmeter

- Accurate flow rate control

- Easy to use digital display

- Easy 3 button control

The control is supplied with a fuse kit rated for the control. The fuse should always be fitted between the battery and controller on the positive cable. Also supplied are four connecting cable lengths (red+black to battery and blue+brown to pump). The control and connecting cables are supplied with bullet connectors to ensure a good quality electrical connection. In order to reduce volt drop the cable should be as short as possible. Please ensure good connections by using the connectors supplied. When using the connecting cables the end without the connectors will require the insulation to be sufficiently stripped back to ensure a good connection with the copper core. For optimum use the controller will need to be calibrated to your system and pole. Follow the instructions as per our quick start guide. In the majority of systems we would expect to see the calibration value between 30 and 55 (this value will vary from system to system). The controller has a number of protections built in. These being:

- DE protection

- Low battery protection

- Reverse polarity protection

- Over current protection (controller must be fitted as per instructions)

- Pump pressure Switch Detection

- Micro DE testing

- Advanced battery management

- Electrical safety test

- Accurate Volt meter

The message DE means the Dead End Detection has shut off the controller and pump. This happens when the controller detects the water flow has been stopped. Reasons for this include: 1) Air bubbles in the system. Run the pump to bleed air from the system. 2) The hose has become disconnected or twisted. Reconnect or untwist the hose. 3) The water tank is empty. Refill your tank. 4) The auto calibration needs to be set. Repeated false dead end detection indicates that the cal value should be increased (less sensitive). Prolonged use of the controller with high calibration may lead to damage to the controller or system. Do not set the cal too high. Setting it higher than necessary places extra strain on both the pump and controller in a DE situation.

The message PS means Pressure Switch. The controller is telling you that the pump's pressure switch has been operated due to a build up of high pressure in your system. The controller is set to work with a pump up to 5.2 litres per minute and 100PSI although it will and can work at higher ranges. Common causes for the pressure switch operating could be:

- A restriction in the system

- Blocked or small water jets (recommended 2mm)

- Grit or dirt trapped in the system

- Air blockage

- Prolonged shut off of the water flow

In very rare cases if you have disconnected the pump pressure switch and are relying only on the controller to detect Dead End when the water flow has been stopped, the following can occur:

- The pump appears to cycle (start then stop)

- The system pressure may build up slightly

- There is a risk of a failure of your hose connections

- The controller and pump are still drawing current

We always recommend that the controller is operated with the pump pressure switch as it will give an additional back up and in the above example will avoid damage to your system as the pressure switch will cut out before dangerous high pressure builds.

The message OC means Over Current. The controller measures the current to the pump (i.e. in the BROWN and BLUE wires) but NOT from the battery (i.e. in the RED and BLACK wires). The controller is designed to shut down with an 'OC' message if current to the pump remains at 7.5A or above. This is to protect the controller from damage. If you see an 'OC' message the controller must be taking more than 7.5A. In this instance it is possible that there is a restriction in the line. This could be down to some dirt, or even using a mismatched fitting or decreasing the jet sizes in a brush. Commonly we have had problems with people using jets that are too small e.g. 1mm instead of 2mm. This restricts flow, builds up pressure in the system and makes the pump work harder than it has to.

The message BAT means Low Battery warning. Below 11.5V the controller flashes 'BAT', warning that the battery is low. Below 11.0V the controller shuts down. This is because, to avoid permanent damage to the battery, the manufacturers recommend that the battery is not run below 11.0V. (If you connect the battery directly to the pump, the system will still work but you may shorten the working life of the battery.) This message may also be displayed if the connections are poor. Do not change or replace the connectors supplied with the controller and keep the wires from the battery to the controller as short as possible.

Water Flow Rate For most cleans we would expect to see water flow up to about 60. Higher flow rates use a higher volume of water with much of the water bouncing off the glass with little cleaning effect. There will be occasions when a higher flow is required. The higher the water flow the higher the current draw from your battery, draining the charge more quickly and reducing the time you can work. There is no need to set the water flow high to generate high pressure, as the hose and brush jets will act to create pressure within your system naturally. The stiffness of the hose will have an impact on calibration as different hoses swell at different rates.