# Smart Battery management

Standard on the V9 and V11 digital controls is a battery management and volt meter function. The  “BAT” message is a warning that voltage has fallen below 11.5V on the V9 controls and 11V on the V11 (manufactured since October 2013)
The Analogue controller uses a Green LED to warn of low Volts
Bat is showing

The battery voltage is available at the touch of a button. Following feedback from distributors and cleaners we re engineered the way the V11 control reads and measures battery voltage. The Controller is continuously checking the available voltage. The Charging controller monitors the state of both leisure and Vehicle batteries. The controller will shut down the system when the battery is too low, below 10.5 volts on the V11, preventing irreversible battery damage.

This gives confidence at the end of the day that you have enough battery power to finish that job without having to swap or recharge the battery.

Low Battery Cut Off Over-ride
Our V11 digital controls feature this as standard simply choose to over ride the low battery cut off. This can be done in the calibration menu at the click of a button. The controller will still display accurate voltage and give a low battery warning.
Note: running your battery below 10.5 volts could cause permanent damage to the battery cells.

Extended Battery Life
Our engineers have excelled by making the V11 Pro digital controller even more energy efficient, allowing
you to work longer on the same charge. Less energy use means:
• longer battery life • less recharging
• efficient energy transfer battery to pump • less wasted energy

The volt meter is standard on our digital controller range. The control will show what it see,s in terms of voltage. Poor quality worn or damaged cable can lead to volt drop along the cable length.
What is volt drop:

Note:
Our digital pump controller has been designed and manufactured to exacting standards and tested fully before leaving the factory. The unit has a warranty for 12 months provided it is fitted and used according to the instructions.

We recommend that in all cases the fuse and the pump pressure switch are fitted, as this gives the best possible protection to the pump, controller and battery, and also allows the most productive and efficient use of your WFP system.